Summary for Part III:

Drag force on submerged body (page 517)

FD=CdAρV22

Where Cd is the drag coefficient, A is the area, V is velocity

Open Channel Flow Hydraulics:

- Uniform flow (or normal flow). (page 440), and Manning equation

Manning equation (SI unit, Q in cubic meter per second):  Q=1nARh23S12

Manning equation (English unit, Q in cfs) Q=1.49nARh23S12

Hydraulic radius:  Rh=A/P, where A is the cross-section area, P is wetted perimeter.

Types of problems:

1.1 Determine wetted parameter (P) and hydraulic radius (Rh) for the following cross sections.

1.2:   Determine velocity and flowrate when other data are given.

  - Determine normal depth ( the depth in Manning equation) when all other data are given.

( When you are asked to determine the super critical flow/subcritical flow, you need to find normal depth from Manning equation to compare with the critical depth)

- Specific Energy (page 677):  Es=y+v22g.

- Critical depth, subpercritical flow, subcritical flow. Critical depth occurs at minimum specific energy

What you need to know:

3.1:  General equation for critical depth (pg 679, eq 15.8): Q2TcgAc3=1

3.2. For rectangular cross-section, critical depth can be determined from Eq 15.13:

yc=(q2g)13,    where q=Q/B

  - If yn > yc, flow is subcritical.

If yn=yc, flow is critical.

If yn<yc, flow is supercritical flow.

- Hydraulic Jump:

Hydraulic jump occurs only when flow changes from supercritical to subcritical flow.

What you need to know:

4.1:  Water depth before and after the hydraulic jump (pg 692, eq 15.27 or 15.28)

y2=y12(1+8Fr12−1)

y1=y22(1+8Fr22−1)

Froude number Fr=v/sqrt(gy)

4.2. Head loss in hydraulic Jump:

hL=(y2−y1)34y1y2

Y1= 0.1 m, v1=10 m/s

Y2=5 m, v2 =0.5 m/s

FIND head loss in the hydraulic jump