CWR4202

Project #1:      Backwater Computation and Analysis (5% of the half semester score)

# ( team of 2 students)

Due Oct 2, 2008

A bridge causeway was built in a river as shown on Figure 1.  Due to the existence of the bridge causeway, the water level increases in the area upstream to the bridge causeway.

Following data are given: Q = 800 m3/s,  channel bottom slope S0 = 0.01,  Manning coefficient n= 0.02

Rectangular cross-section: 	bottom width = 50 m.

Depth at the bridge causeway y0= 7 m.

Bridge deck

Figure 2: Schematic diagram: effects of a bridge pier on river flow under flood condition

---  normal depth,   __  backwater profile.

1)

Develop a computer program to determine the backwater profile. The program should include the   following component:

- Determine normal depth (yn)

- Determine the flow condition ( subcritical or critical flow?)

- Determine the depth of backwater profile y(x)

- Graphic presentation of bottom elevation ‘zb’, back water surface elevation ’ z’ along the channel.

2.) A house is located 1000 m upstream of the bridge. Before the construction of the bridge, the elevation of house foundation is 0.5 m above the surface of the normal river flow. For the conditions given in (1), will the house be flooded after the construction of the bridge?

3) Write a project report. The report should include the following sections:

- Introduction ( description of the project, available data, and requirement)

- Methodology

( equations and solution procedures for yn, yc, y …)

- Results and analysis ( details of solutions for each questions)

- Attachment ( a hard copy of computer program,  and e-mail a electronic copy to me)

f). Report should be written by using word processing software such as MS Word.